# Scarlet First-Fire Checklist

## Fuel

- Jerry can acquired and filled

- Fresh petrol in tank

- Fuel pump primes

- No fuel leaks

### Injectors

- Measure injector resistance

- Confirm injector type:

  - High impedance (~12-16 Ω)

  - Low impedance (~2-4 Ω)

- Verify MS1 injector current limiting/PWM settings match injector type

- Confirm injector flow setting = 190 cc/min (starting point)

- Note Bosch part number for future reference

## ECU & Sensors

- CLT reading sensible

- IAT reading sensible

- TPS calibrated and reading correctly

- MAP reading sensible with ignition on

- Stable RPM signal while cranking

- Fuel pump output functioning correctly

## Base Tune

- Fuel algorithm = Speed Density

- Idle target = 950 rpm

- Idle MAP estimate = 45 kPa

- EGO correction = OFF

- Save current MSQ before making changes

- Create Git repo for MSQ files (if feeling keen)

## Trigger Setup (Highest Priority)

Using flywheel TDC mark:

- Set Fixed Advance = 0°

- Crank engine

- Adjust Trigger Angle / Offset until timing light aligns with TDC

- Set Fixed Advance = 10°

- Confirm timing moves exactly 10°

- Re-enable table timing

### Wheel Decoder

Leave alone until proven otherwise:

- Trig Pos A = 3

- Trig Return A = 8

- Trig Pos B = 21

- Trig Return B = 26

## Ignition

- Confirm spark on all cylinders

- Verify coil wiring

- Verify firing order (1-3-2-4)

- Resist urge to "improve" the spark table before first start

## First Crank

Success criteria:

- Attempts to fire

- Doesn't kick back

- RPM remains stable

- No ECU resets

## First Run

- Oil pressure present

- No unusual mechanical noises

- No fuel leaks

- No coolant leaks

- AFR appears believable

- Charging voltage sensible

- Coolant temperature rises normally

## Warm Idle

- Reach operating temperature

- Record actual idle MAP

- Record actual idle AFR

- Aim for ~950 rpm warm idle

- Adjust throttle stop before chasing tune issues

## After It Runs

- Save MSQ immediately

- Commit MSQ to Git with notes

- Enable EGO later

- Start refining VE table

- Compare against proven Alfa boxer spark maps

- Only then begin spark optimization

## Mission Objective

Not:

Make perfect tune.

But:

Add petrol.
Confirm timing.
Make boxer noises.

🏁😁