# Rear Structure Stabilisation

## CIO frame horns

Clean and degrease

Wire-brush surfaces

Brush on Buzzweld CIO

Let dry fully

## CIO torsion tubes

Degrease thoroughly

Remove loose material

Apply CIO evenly

Dry before handling

## CIO spring plates

Remove if needed

Clean edges and faces

Apply CIO coating

Cure properly

## CIO axle tubes

Degrease exterior

Remove contaminants

Brush CIO over tube

Allow to set

## CIO gearbox side plates

Clean plates

Remove residue

Apply CIO

Dry before assembly

# Gearbox Assembly & Installation

## Assemble gearbox axle tubes & boots

Fit boots

Assemble plates

Set droop angle

Prep for install

## Hang gearbox

Lift into place

Bolt to horns

Torque mounts

Verify alignment

## Refit spring plates

Align

Fit hardware

Set ride height

Torque safely

# Rear Brakes & Rear Wheels

## Build Type 3 rear brakes

Install plates

Fit shoes/cylinders

Adjust hardware

Prepare for bleed

## Fix rear bead leaks

Pop beads

Strip bead seats

Clean thoroughly

Reseal and refit tyres

## Refit rear wheels

Mount wheels

Torque nuts

Check bead seal

Prepare for rolling

# Front Wheels & Mobility

## Remove old front tyres

Break beads

Remove tyres

Inspect rims

Clean surfaces

## Prep front wheels

Strip bead seats

Clean valve holes

Remove corrosion

Ready for paint

## Paint front wheels

Hammerite Smooth Red

Two medium coats

Avoid bead seat

Cure 24–48 hours

## Fit front tyres & valves

Install valves

Mount tyres

Seat beads

Inflate properly

## Refit front wheels

Install on hubs

Torque nuts

Check rolling

Confirm clearance

# Fibreglass Repairs

## Fill tail light holes

Glass backing patch

Fill flush

Block shape

Ready for primer

## Fill bonnet handle holes

Glass from rear

Fill front

Sand level

Primer-ready

## Repair bonnet nose crack

V-groove crack

Glass reinforcement

Filler skim

Shape nose

## Spot-fill worst gel coat crazing

Identify worst zones

Skim filler

Block lightly

Limit to 1 weekend

## Local filler primer

High-build primer

Only repaired panels

Light block

Stop before chasing perfection

# Red Oxide

## Prep shell for red oxide

Mask components

Degrease

Light scuff

Ready for coating

## Apply full-shell red oxide

Roll/brush evenly

Cover GRP + filler

Patch thin spots

Cure completely

# Engine Install & Wiring

## Source M10 studs

Find 10.9 grade

Confirm M10x1.5

Check lengths

Order parts

## Fit engine studs

Clean threads

Medium Loctite

Install studs

Verify protrusion

## Mount engine to gearbox

Mate engine

Align clutch

Torque bolts

Check clearance

## Fuel system completion

Run feed line

Check return

Leak test

Prime system

## Cooling system plumbing

Connect hoses

Secure with P-clips

Check rub points

Fill coolant

## Essential wiring

Kill switch

Starter circuit

Fan/temp switch

ECU power/grounds

## Sensors & wideband

Install wideband

Wire interface box

Connect ECU

Check signals

## First fire

Load base map

Check timing

Start engine

Heat cycle

# Road Capability & RTW Readiness

## Brake bleed & test

Bleed system

Check pedal feel

Re-bleed after bedding

Inspect leaks

## Road shakedown

Short local runs

Check temps

Watch AFRs

Build fix list

## Balance wheels

Perform balancing

Check vibration

Adjust if needed

Prepare for trip

## Alignment (basic)

Set front toe

Check steering

Not final spec

Safe for road

## RTW prep & loadout

Pack spares

Print wiring map

Check tools

Final visual check

## Drive to Bridlington

Check fluids

Early start

Monitor temps

Arrive beach-ready